# IoT Patient Health Monitor

Friend Setup Guide — From Zero to Running

ESP32 + MAX30102 + DS18B20  →  FastAPI Backend  →  React Dashboard

# Overview

This guide walks you through setting up the entire health monitoring system on your machine. By the end, your ESP32 will be reading heart rate, SpO2, and temperature — sending it to a backend server — and displaying it live on a dashboard in your browser.

What you will need:

ESP32 development board (ESP32-WROOM-32)

MAX30102 sensor (heart rate + SpO2)

DS18B20 waterproof temperature sensor

4.7kΩ resistor (for DS18B20)

Jumper wires + breadboard

USB cable to connect ESP32 to your PC

PC running Linux, macOS, or Windows with Python 3.10+ and Node.js 18+

# PART 1 — Get the Code

## Step 1: Install Git (if not installed)

Open your terminal and run:

sudo apt install git -y          # Linux (Debian/Ubuntu/Kali)

# macOS: git comes pre-installed

# Windows: download from https://git-scm.com

## Step 2: Clone the Repository

Run this in your terminal:

git clone git@github.com:nischal-khadka/health-monitor.git

cd health-monitor

NOTE: If you don't have SSH set up, use HTTPS instead:

git clone https://github.com/nischal-khadka/health-monitor.git

You should now see this folder structure:

health-monitor/

├── backend/       ← FastAPI server

├── frontend/      ← React dashboard

├── firmware/      ← ESP32 code

└── README.md

# PART 2 — Hardware Setup (ESP32 + Sensors)

## Step 3: Wire the MAX30102 Sensor (Heart Rate + SpO2)

Connect the MAX30102 to the ESP32 using jumper wires:

| MAX30102 Pin | ESP32 Pin |
| --- | --- |
| VIN | 3.3V |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |

## Step 4: Wire the DS18B20 Sensor (Temperature)

| DS18B20 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| DATA | GPIO 4 |

NOTE: Place a 4.7kΩ resistor between the DS18B20 DATA pin and VCC pin. This is required — without it the sensor will not work.

# PART 3 — Backend Setup

## Step 5: Install Python Dependencies

Make sure Python 3.10 or higher is installed:

python3 --version

Install the required packages:

cd backend

pip install -r requirements.txt

NOTE: The requirements include bcrypt==4.0.1 which is required. If you see a bcrypt error when registering, make sure you ran pip install -r requirements.txt from inside the backend folder.

## Step 6: Set Up PostgreSQL Database

Install PostgreSQL if not installed:

sudo apt install postgresql -y

sudo systemctl start postgresql

Create the database and user:

sudo -u postgres psql -c "CREATE DATABASE health_monitor;"

sudo -u postgres psql -c "CREATE USER health_user WITH PASSWORD 'health_pass123';"

sudo -u postgres psql -c "GRANT ALL PRIVILEGES ON DATABASE health_monitor TO health_user;"

sudo -u postgres psql -c "ALTER DATABASE health_monitor OWNER TO health_user;"

## Step 7: Create the Database Tables

cd backend

python3 -c "from app.db.database import Base, engine; from app.models.vitals import Vital; Base.metadata.create_all(engine); print('Tables created')"

NOTE: Run this as one single line in your terminal.

## Step 8: Start the Backend Server

cd backend

uvicorn app.main:app --reload --host 0.0.0.0 --port 8000

Open your browser and go to:

http://localhost:8000/docs

You should see the Swagger API documentation page with 3 endpoints. This confirms the backend is running correctly.

# PART 4 — Frontend Dashboard Setup

## Step 9: Install Node.js (if not installed)

Check if Node.js is installed:

node --version

If not installed:

sudo apt install nodejs npm -y    # Linux

# Or download from https://nodejs.org  (Windows/macOS)

## Step 10: Install Frontend Dependencies

cd frontend

npm install

## Step 11: Start the Dashboard

cd frontend

npm run dev

Open your browser and go to:

http://localhost:5173

You will be redirected to the login page. Click Register to create your account, then sign in. You will see the dashboard with Heart Rate, SpO2, and Temperature cards.

# PART 5 — ESP32 Firmware Setup

## Step 12: Install PlatformIO

Install PlatformIO CLI:

pip install platformio

Verify installation:

pio --version

## Step 13: Find Your PC's IP Address

You need your PC's local IP so the ESP32 can send data to it.

hostname -I          # Linux/macOS

ipconfig             # Windows

Look for something like 192.168.1.x — that is your IP.

## Step 14: Edit the Firmware Configuration

Open the file:

firmware/src/main.cpp

Find and edit these 3 lines at the top:

const char* WIFI_SSID    = "YOUR_WIFI_NAME";

const char* WIFI_PASS    = "YOUR_WIFI_PASSWORD";

const char* API_ENDPOINT = "http://192.168.1.x:8000/api/vitals/";

NOTE: Replace YOUR_WIFI_NAME and YOUR_WIFI_PASSWORD with your actual WiFi credentials. Replace 192.168.1.x with your actual PC IP from Step 13.

## Step 15: Flash the ESP32

Connect your ESP32 to your PC via USB cable, then run:

cd firmware

pio run --target upload

NOTE: First run will download the ESP32 toolchain and libraries automatically. This may take 5-10 minutes.

## Step 16: Monitor Serial Output

pio device monitor

You should see output like this:

WiFi connected: 192.168.1.x

HR: 72 bpm | SpO2: 98% | Temp: 36.50 C

POST http://192.168.1.x:8000/api/vitals/ -> HTTP 201

HTTP 201 means data was saved successfully.

# PART 6 — Test Everything Together

## Step 17: Run All Three Components

You need 2 terminals open at the same time:

| Terminal | Command |
| --- | --- |
| Terminal 1 (Backend) | cd backend && uvicorn app.main:app --reload --host 0.0.0.0 --port 8000 |
| Terminal 2 (Frontend) | cd frontend && npm run dev |

Then make sure your ESP32 is powered on and connected to WiFi.

## Step 18: Create Your First Account

Open your browser and go to the register page:

http://localhost:5173/register

Fill in your username, email, and password then click Register. You will be redirected to login automatically.

After logging in, add your first patient via the API docs:

http://localhost:8000/docs

Use the POST /api/patients/ endpoint to add a patient (name, age, gender, ward).

## Step 19: Test Without ESP32 (Optional)

Send a fake reading to verify the full stack works (replace 1 with your actual patient ID):

curl -X POST http://localhost:8000/api/vitals/ \

-H 'Content-Type: application/json' \

-d '{"patient_id":1,"heart_rate":75,"spo2":98,"temperature":36.5}'

Then open http://localhost:5173 — you should see the values appear on the dashboard within 5 seconds.

# Troubleshooting

## Backend won't start

Make sure PostgreSQL is running: sudo systemctl start postgresql

Make sure you ran the table creation command in Step 7

Check the .env file in backend/ has the correct DATABASE_URL

## Registration fails with error

Make sure you ran: pip install -r requirements.txt from inside the backend/ folder

The file requires bcrypt==4.0.1 — newer versions break password hashing

Restart the backend after installing dependencies

## ESP32 won't connect to WiFi

Double-check WIFI_SSID and WIFI_PASS in main.cpp

Make sure your PC and ESP32 are on the same WiFi network

Try moving the ESP32 closer to the router

## ESP32 connects but dashboard shows no data

Check API_ENDPOINT IP matches your PC's IP (hostname -I)

Make sure backend is running on port 8000

Check serial monitor — it should show HTTP 201, not an error

## Dashboard not loading

Make sure npm run dev is running in the frontend folder

Open http://localhost:5173 (not 8000)

# Quick Reference

| Service | URL / Command |
| --- | --- |
| Backend API | http://localhost:8000 |
| API Docs (Swagger) | http://localhost:8000/docs |
| Frontend Dashboard | http://localhost:5173 |
| Database | postgresql://health_user:health_pass123@localhost/health_monitor |
| Serial Monitor | pio device monitor (in firmware folder) |

Good luck! — The system is designed to work out of the box once configured.